Ain Shams University

Faculty of Computer and Information Sciences

Computer Science Program

Real-Time Epileptic Seizure Detection from Overhead Hospital-Bed Video

Graduation Project Report

By

Omar Salama

Under Supervision of

[Supervisor Name]

Faculty of Computer and Information Sciences

Ain Shams University

2026

# Acknowledgement

The author gratefully acknowledges the contributions of the research community whose open-source implementations and published checkpoints made this work possible. Particular thanks are extended to the authors of VSViG (Xu et al., ECCV 2024) for their public checkpoint release and technical correspondence, and to the authors of the Cross-Joint Attention framework (Zamzam et al., 2026) for their published model and training methodology.

The project benefited substantially from the open availability of the HuggingFace Transformers library, the ONNX Runtime, and the lightweight-human-pose-estimation implementation by Daniil Osokin.

# Abstract

Epileptic seizure detection in inpatient Epilepsy Monitoring Units (EMUs) currently requires continuous manual video review, which is labour-intensive and prone to delay. This project presents a real-time, video-only seizure detection system designed for unobtrusive overhead camera deployment, requiring no wearables or contact sensors.

The system combines a lightweight skeleton-based temporal risk branch (VSViG/ProtoGCN, 6 FPS, ~2.5 ms/inference) with a high-capacity Cross-Joint Attention classifier driven by a frozen ViViT-B/Kinetics400 feature extractor. A novel series gate architecture selectively trusts or blends these two complementary signals to produce binary clinical alerts (NORMAL / SEIZURE) with a 30-second temporal latch, enabling early pre-clinical alerting while suppressing spurious motion artefacts.

The deployment architecture is a hybrid C++/Python pipeline. The high-frequency VSViG branch and gate logic run natively in C++ via ONNX Runtime. The ViViT transformer runs asynchronously in a Python sidecar process, exchanging data with the C++ engine through a Windows Named Pipe. The system achieves a steady-state throughput of 37.21 FPS on RTX-class hardware.

On a 952-segment held-out test set derived from 14 EMU patients (33 seizure events), the system achieves AUROC 0.9669, AUPRC 0.9429, F1 0.9018, and recall 0.9845. In a 10-video clinical runtime evaluation, the system detects 9 of 10 targeted events with a mean Latency to Clinical Onset (LCO) of −3.94 seconds (pre-clinical alerting) and a mean Latency to EEG Onset (LEO) of +5.86 seconds. The false alert rate is 5.3 per hour on the evaluation set.

This report documents the full system design, training methodology, experimental history, and evaluation results, in conformance with the Ain Shams University Graduation Project guidelines.

# List of Abbreviations

| Abbreviation | Meaning |
| --- | --- |
| AP | Accumulative Probability |
| AUPRC | Area Under Precision-Recall Curve |
| AUROC | Area Under Receiver Operating Characteristic Curve |
| CJ | Cross-Joint (Attention Head) |
| CNN | Convolutional Neural Network |
| CTR-GCN | Channel-wise Topology Refinement Graph Convolutional Network |
| DXA | Document Extended Attribute (Word measurement unit; 1440 DXA = 1 inch) |
| EEG | Electroencephalogram |
| EMU | Epilepsy Monitoring Unit |
| FAR | False Alert Rate |
| FPS | Frames Per Second |
| GCN | Graph Convolutional Network |
| GPU | Graphics Processing Unit |
| HuggingFace | Open-source ML model hub and library |
| IPC | Inter-Process Communication |
| LCO | Latency to Clinical Onset |
| LEO | Latency to EEG Onset |
| LoRA | Low-Rank Adaptation |
| ML | Machine Learning |
| MLA | Multi-Latent Attention |
| ONNX | Open Neural Network Exchange |
| OpenPose | Real-time multi-person skeleton estimation system |
| PAF | Part Affinity Field |
| ProtoGCN | Prototype Graph Convolutional Network |
| RMSE | Root Mean Square Error |
| ROI | Region of Interest |
| VSViG | Video-Based Seizure detection via Skeleton-Based Spatiotemporal ViG |
| ViG | Vision GNN (Graph Neural Network) |
| ViViT | Video Vision Transformer |
| YAML | YAML Ain't Markup Language |

# Chapter 1: Introduction

## 1.1 Motivation

Epilepsy is one of the most prevalent serious neurological disorders worldwide, affecting approximately 50 million people. Accurate seizure monitoring is essential for both diagnosis and for optimising anti-epileptic drug therapy. In specialised Epilepsy Monitoring Units, patients are admitted for extended inpatient video-EEG telemetry, during which clinical staff must continuously observe video footage and EEG traces to identify and characterise seizure events.

This monitoring process is inherently labour-intensive. A single patient may be observed for multiple days, yet seizures may last only seconds and can occur at any time, including overnight. Manual review imposes a significant staff burden and introduces human error, particularly during periods of reduced alertness. The consequence of a missed or delayed seizure detection can be clinically significant: delayed medical intervention, injury risk from unwitnessed convulsions, and incomplete semiology documentation.

Computer-assisted seizure detection from bedside video offers a compelling solution. A system that can continuously process video streams, identify the motor manifestations of seizures, and generate automated alerts would reduce staff fatigue, accelerate response times, and enable reliable overnight monitoring. Importantly, a video-only approach requires no patient contact — no electrode application, no wearable devices — making it practical for immediate routine deployment in any EMU equipped with an overhead camera.

Despite the clear clinical need, real-time video-based seizure detection remains an open research problem. The core challenge is discriminating the characteristic but variable motor patterns of seizures — which include focal limb jerking, generalised tonic-clonic convulsions, automatisms, and dystonic posturing — from a broad set of superficially similar patient movements such as repositioning, scratching, coughing, and voluntary limb activity. This discrimination must be performed at clinical latencies (ideally several seconds before clinical onset becomes visible) and at throughput rates sufficient for continuous operation.

## 1.2 Problem Definition

The problem addressed by this project is the automated real-time detection of motor seizures from an overhead fixed-camera hospital-bed video stream, without the use of EEG, wearables, or any other sensor modality beyond RGB video.

The specific technical challenges are:

- Temporal resolution: Seizure motor onset may precede full clinical expression by several seconds. A useful clinical system must alert before clinical onset (negative Latency to Clinical Onset), ideally while also leading EEG onset by a modest margin.

- Motion discrimination: The system must distinguish seizure motor patterns from a large and variable set of normal patient movements, under varying illumination, blanket occlusion, and patient position.

- Real-time throughput: The system must process video at frame rates commensurate with standard EMU camera framerates (~25-30 FPS) without hardware requirements that preclude routine hospital deployment.

- Robustness: The system must operate continuously for extended periods without failure, including graceful handling of pose estimation failures, model latency spikes, and process interruptions.

- Clinical interpretability: The system must produce binary categorical outputs (NORMAL / SEIZURE) with a temporal latch duration consistent with clinical response workflows.

## 1.3 Objective

The primary objective of this project is to design, implement, evaluate, and document a real-time hybrid C++/Python video-based seizure detection pipeline deployable in an EMU context. Specific objectives are:

- Reproduce and adapt the VSViG skeleton-based seizure detection model (Xu et al., ECCV 2024) as a lightweight, synchronous temporal risk branch capable of operating at 6 FPS with sub-3 ms inference time.

- Integrate the Cross-Joint Attention classifier (Zamzam et al., 2026) using frozen ViViT-B/Kinetics400 features as the primary segment-level seizure classifier, trained via knowledge distillation to enable deployment-independent weights.

- Design and implement a novel series gate fusion architecture that combines the two branches to achieve pre-clinical alerting (negative LCO) while maintaining a clinically acceptable false alert rate.

- Build a hybrid C++/Python deployment pipeline that satisfies real-time throughput requirements (>30 FPS) through native ONNX inference for the high-frequency branch and asynchronous IPC for the transformer branch.

- Evaluate the complete system on a held-out 952-segment test set and a 10-video clinical timing evaluation, and document all experimental history including failed approaches.

## 1.4 Document Organization

This document is organized as follows:

Chapter 2: Background provides a review of the relevant literature in video-based seizure detection, skeleton-based action recognition, and video transformer architectures. It surveys the key methods that directly informed the system design.

Chapter 3: Analysis and Design presents the system architecture, functional and non-functional requirements, and the design of the deployment pipeline. Mermaid-compatible architecture diagrams are included for key components.

Chapter 4: Implementation documents the technical implementation of each system component in detail, including the C++ native runtime, the Python sidecar, the series gate logic, and the training methodology for both the VSViG and CJ branches.

Chapter 5: Experimental Methodology, Results, and Evaluation presents the evaluation protocols, all segment-level and clinical runtime metrics, and a comprehensive experimental history of all approaches investigated during development, including failed trials.

Chapter 6: Conclusions and Future Work summarises the project outcomes, discusses limitations of the current system, and proposes directions for future research.

# Chapter 2: Background

## 2.1 Clinical Context: Seizure Monitoring in the EMU

An Epilepsy Monitoring Unit is a specialised inpatient facility in which patients with refractory or undiagnosed seizures undergo prolonged video-EEG telemetry. The gold-standard clinical workflow combines continuous digital EEG with synchronised video capture, enabling clinicians to correlate electrographic seizure activity with its clinical motor manifestations.

Two key temporal landmarks define a seizure event. The EEG onset is the time at which rhythmic ictal activity first appears on the scalp EEG recording. The clinical onset is the time at which visible motor symptoms appear on the video. For focal motor seizures, the clinical onset typically lags EEG onset by several seconds. A video-based system that detects motor manifestations cannot logically precede EEG onset; however, it can precede the full clinical expression of the seizure. A system that raises an alert before clinical onset is said to achieve pre-clinical alerting, quantified as a negative Latency to Clinical Onset.

Two operational metrics define clinical usefulness: Latency to Clinical Onset (LCO) and False Alert Rate (FAR). Negative LCO indicates the system alerts before the clinician would observe overt motor activity. FAR, measured in alerts per hour of non-ictal video, quantifies the rate at which staff are disrupted by false alarms.

## 2.2 Skeleton-Based Seizure Detection: VSViG

Xu et al. (2024) introduced VSViG (Video-based Seizure detection via Skeleton-based Spatiotemporal Vision GNN), a lightweight real-time seizure detection framework published at ECCV 2024. VSViG addresses the real-time constraint by operating exclusively on skeleton keypoints rather than raw video, avoiding the prohibitive inference cost of large video transformers.

The VSViG architecture extracts OpenPose-18 keypoints at 6 FPS, selects 14 clinically relevant joints, and constructs Gaussian-weighted 32×32 RGB patches centered at each joint. These joint-image tubelets are processed by a ProtoGCN (Prototype Graph Convolutional Network) variant that models both spatial joint relationships and temporal dynamics across a 30-frame (5-second) window. The model outputs a single seizure risk probability per window.

On the authors' 14-patient dataset using a 70/30 period split, VSViG achieves a mean LEO of +5.1 seconds, a mean LCO of −13.1 seconds, an RMSE of 5.9%, and a zero false detection rate on their held-out test set. The model contains 5.4 million parameters and achieves approximately 2.5 ms inference time on modern GPU hardware.

The primary limitation of VSViG as a standalone system is its inability to precisely classify seizure segments at the window level: the accumulative probability approach produces wide alert regions that may include pre-ictal and post-ictal periods. The model functions well as a sensitive temporal alarm but requires additional discriminative components to suppress false alarms.

## 2.3 Cross-Joint Attention for Seizure Detection

Zamzam et al. (2026) introduced a Cross-Joint Attention framework for video-based seizure detection, exploiting the rich motion representations available in large video transformers pre-trained on action recognition datasets. The key insight is that a ViViT model (Video Vision Transformer, Arnab et al., 2021) pre-trained on Kinetics-400 encodes high-level spatiotemporal features relevant to human motion without requiring any seizure-specific fine-tuning.

The Cross-Joint methodology extracts per-joint video tubelets centred at each of 14 skeleton keypoints and passes them independently through a frozen ViViT backbone, obtaining one 768-dimensional token per joint per segment. A Joint Transformer Classifier (JointTransformerClassifier) then attends over these 14 joint tokens, using sinusoidal position embeddings derived from keypoint coordinates to provide joint-identity context, and outputs a single binary seizure probability.

Zamzam et al. report AUROC 0.921 ± 0.012 and F1 0.841 ± 0.019 for the frozen ViViT variant, and AUROC 0.942 ± 0.008 and F1 0.875 ± 0.014 for the full LoRA fine-tuning variant, on a multi-patient EMU dataset. Training uses Binary Cross-Entropy with Logits Loss and AdamW optimisation.

The primary limitation of the Cross-Joint approach as a standalone runtime system is its latency. Processing a 5-second segment through the ViViT backbone requires approximately 1,050 ms on GPU hardware, meaning the system produces at most one decision every five seconds. This is insufficient for pre-clinical alerting at the frame level and motivates the integration of a fast temporal branch.

## 2.4 Video Vision Transformer (ViViT)

ViViT (Arnab et al., 2021) is a pure Transformer architecture for video classification. The model tokenises a video clip by partitioning it into non-overlapping spatiotemporal tubes, each mapped to a sequence token via a linear projection. Multiple factorised self-attention variants are available; the ViViT-B/16x2 model used in this project employs a factorised encoder with alternating spatial and temporal attention layers.

When pre-trained on Kinetics-400, ViViT learns rich representations of human motion, object interaction, and scene dynamics. The representations generalise well to downstream tasks through feature extraction (frozen backbone) and fine-tuning. The HuggingFace model hub provides a public checkpoint of ViViT-B/16x2-Kinetics400 (google/vivit-b-16x2-kinetics400) which is used directly in this project.

## 2.5 Skeleton-Based Action Recognition: CTR-GCN

Channel-wise Topology Refinement Graph Convolutional Network (CTR-GCN, Chen et al., 2021) is a state-of-the-art skeleton-based action recognition model that dynamically refines the GCN topology per channel. It supports multi-stream processing of joint, bone, and motion features, with ensemble combination typically improving single-stream performance.

CTR-GCN was evaluated in this project as a skeleton-only baseline to quantify the contribution of frozen ViViT features. Results (Section 5.2.2) demonstrate that CTR-GCN achieves substantially lower classification performance than the Cross-Joint framework, motivating the use of video-level features over skeleton topology alone.

## 2.6 Knowledge Distillation

Knowledge distillation (Hinton et al., 2015) is a training paradigm in which a student model is trained to match the soft output distribution of a larger or more accurate teacher model, in addition to the hard ground-truth labels. The combination of soft teacher logits and hard labels regularises the student and typically produces better-calibrated outputs than hard-label training alone.

In this project, distillation is applied to train locally owned CJ classification heads from the official Zamzam et al. checkpoint. The distillation mix is 0.5 soft teacher labels and 0.5 hard binary labels. This enables deployment of the system without dependence on the released checkpoint weights while achieving competitive classification performance.

## 2.7 ONNX Runtime and Deployment

The Open Neural Network Exchange (ONNX) format provides a standardised intermediate representation for ML models that enables cross-framework deployment. ONNX Runtime is a high-performance inference engine for ONNX models supporting CPU and GPU (CUDA) execution providers. In this project, three ONNX models are deployed in C++: the OpenPose pose estimation model, the VSViG/ProtoGCN model, and the CJ classification head.

ONNX deployment introduces a critical requirement: explicit parity verification between the PyTorch training model and the exported ONNX graph. This project encountered and resolved a silent ONNX parity failure (Section 5.3.4) that produced wrong but non-zero outputs, underscoring the necessity of fixture-based parity testing for every exported model.

# Chapter 3: Analysis and Design

## 3.1 System Overview

### 3.1.1 Proposed Architecture

The proposed system fuses two complementary seizure detection branches through a novel series gate architecture. The overall design is motivated by the complementary failure modes of the two branches: VSViG provides high temporal resolution but limited discriminative power; the Cross-Joint transformer provides strong segment-level discrimination but operates with 5-second latency.

The series gate resolves this tension by defining three operating regimes based on the CJ confidence level. When CJ confidence is high (>0.50), the gate trusts the CJ decision without blending. When CJ confidence falls in an uncertain range (0.20–0.50), the gate linearly blends CJ and VSViG signals, allowing VSViG to rescue equivocal CJ predictions. When CJ confidence is very low (<0.20), the gate trusts the CJ low-confidence signal directly. This design ensures that the VSViG branch can only rescue towards a seizure decision, not away from one, preserving specificity.

The deployment architecture separates high-frequency and low-frequency processing. The VSViG branch, gate logic, alert latch, and output logging run natively in C++ via ONNX Runtime, ensuring the main monitoring loop is never blocked by Python interpreter overhead. The ViViT transformer runs in an isolated Python process, pushing segment scores asynchronously via Windows Named Pipe IPC.

### 3.1.2 Functional Requirements

The following functional requirements define the system behaviour:

- FR-1: The system shall accept a continuous video stream from a standard EMU overhead camera as its sole input sensor.

- FR-2: The system shall extract 14 clinically relevant skeleton keypoints from each input frame using an OpenPose-18 model.

- FR-3: The system shall maintain a 30-frame rolling keypoint buffer and compute a per-frame VSViG seizure risk probability at 6 FPS.

- FR-4: The system shall asynchronously extract ViViT features from 5-second keypoint-centred video tubelets and compute a Cross-Joint seizure probability every 5 seconds.

- FR-5: The system shall combine VSViG and CJ probabilities through a series gate to produce a binary clinical status output (NORMAL or SEIZURE) at every frame.

- FR-6: The system shall latch the SEIZURE status for a minimum of 30 seconds once an alarm threshold is crossed.

- FR-7: The system shall degrade gracefully to VSViG-only operation if the CJ IPC connection becomes stale (>5.5 s without a new CJ packet).

- FR-8: The system shall log per-frame alert data to a CSV file including status, gate score, VSViG risk, CJ probability, and CJ packet age.

- FR-9: The system shall support offline operation in air-gapped hospital networks through local model file deployment and HuggingFace offline mode.

- FR-10: The system shall achieve a steady-state processing throughput of at least 30 FPS on an NVIDIA RTX-class GPU.

### 3.1.3 Non-Functional Requirements

| Requirement | Description | Target |
| --- | --- | --- |
| Performance | Steady-state processing rate on RTX-class GPU | ≥30 FPS |
| Reliability | Continuous uninterrupted operation during a monitoring session | No unhandled crashes |
| Latency (LEO) | Mean detection latency relative to EEG onset | ≤10 s |
| Latency (LCO) | Mean detection latency relative to clinical onset | <0 s (pre-clinical) |
| Specificity | False alert rate on non-ictal video | <10 / hr |
| Sensitivity | Segment-level recall on held-out test set | ≥0.95 |
| Portability | Minimum GPU VRAM required | 6 GB |
| Security | Patient data must not be transmitted externally | Local processing only |

### 3.1.4 System Users

A. Intended Users: The primary users of the system are EMU nurses and neurophysiology technicians who monitor live patient video feeds. The system operates autonomously and surfaces alerts through its SEIZURE status output; users interact with the alert output rather than with model internals. Secondary users include biomedical engineers responsible for installation and maintenance.

B. User Characteristics: Nursing and technician staff require no machine learning expertise. They interact with the alert status and log files only. Biomedical engineers responsible for deployment require familiarity with Windows command-line tools and ONNX model file management.

## 3.2 System Analysis and Design

### 3.2.1 Use Case Diagram

Primary actors: EMU Nurse (observes alerts), Biomedical Engineer (installs and configures system), Patient (monitored passively).

Key use cases:

- UC-1: Monitor patient video stream (Nurse observes continuous status output; system generates NORMAL/SEIZURE)

- UC-2: Receive seizure alert (Nurse is notified by SEIZURE status; responds to patient)

- UC-3: Install and configure system (Biomedical Engineer deploys ONNX models, configures thresholds)

- UC-4: Review alert log (Clinician reviews per-frame CSV log for post-event analysis)

- UC-5: Run evaluation (Researcher runs evaluate_runtime.py or evaluate_segments.py to reproduce benchmarks)

### 3.2.2 Component Class Overview

The primary software components and their roles are:

| Component | Language | Role |
| --- | --- | --- |
| full_runtime.cpp | C++ | Main runtime: video decode, pose, VSViG, IPC, gate, CSV logging |
| seizure_gate.hpp | C++ | Series gate logic, alert latch, threshold constants |
| main.cpp | C++ | Gate replay, gate evaluate, gate bench CLI |
| vivit_ipc_server.py | Python | ViViT token extraction, Named Pipe producer |
| pose.py | Python | PyTorch pose model loader and keypoint extractor |
| tubelet_builder.py | Python | Joint tubelet construction from keypoints and video frames |
| models.py | Python | JointTransformerClassifier, LoRA ViViT wrapper |
| evaluate_segments.py | Python | Segment-level AUROC/AUPRC evaluation |
| evaluate_runtime.py | Python | Clinical timing (LEO/LCO) and FPS evaluation |

### 3.2.3 Sequence Diagram: Alert Generation

The following sequence describes one complete alert cycle from frame acquisition to CSV log output:

- C++ runtime reads frame from OpenCV VideoCapture.

- If frame index satisfies sampling stride, frame is resized and passed to OpenPose ONNX (C++).

- OpenPose heatmaps are decoded to 14 keypoints. Gaussian patches are extracted and buffered.

- When 30-frame buffer is full and inference stride is satisfied, VSViG ONNX produces a logit; sigmoid converts to current_risk.

- Separately, Python sidecar reads the same video, accumulates 30 sampled frames, builds joint tubelets, runs ViViT, and writes token + position payload to Named Pipe.

- C++ IPC thread reads payload, runs cj_final.onnx, pushes {prob, time_sec} to pending queue.

- C++ main loop promotes pending CJ sample to active sample when its time_sec <= current video time.

- SeizureGate::decide_from_signals applies series gate formula using current_risk and active cj_prob.

- Result (status, score, source, alert_latched) is appended to CSV.

### 3.2.4 Gate State Machine

The alert logic implements the following state transitions:

| Current State | Condition | Next State |
| --- | --- | --- |
| INITIALISING | No CJ sample received yet | INITIALISING |
| INITIALISING | First CJ sample received | NORMAL or SEIZURE |
| NORMAL | Gate score < 0.49 | NORMAL |
| NORMAL | Gate score >= 0.49 | SEIZURE (latch starts) |
| SEIZURE | Latch timer < 30s | SEIZURE |
| SEIZURE | Latch timer >= 30s and gate score < 0.49 | NORMAL |

# Chapter 4: Implementation

## 4.1 Repository Structure

The repository implements a video-based seizure detection runtime and evaluation environment. The top-level structure is as follows:

| Path | Purpose |
| --- | --- |
| configs/default.yaml | Runtime model paths and sampling parameters |
| evaluation/ | Segment-level and runtime clinical evaluation scripts |
| model_weights/ | Deployed ONNX and PyTorch model artifacts |
| runtime/src/full_runtime.cpp | Main C++ runtime executable |
| runtime/src/main.cpp | Gate replay, evaluate, and bench CLI |
| runtime/include/seizure_gate.hpp | Series gate, latch, and threshold constants |
| runtime/vivit_ipc_server.py | Python ViViT IPC sidecar |
| runtime/utils/ | Python pose, patch, tubelet, and path utilities |
| vendor/joint-attention-seizure-detection/ | CJ and ViViT training code |
| vendor/lightweight-human-pose-estimation.pytorch/ | OpenPose-style PyTorch pose model |

## 4.2 Frame Acquisition and Sampling

The C++ runtime opens the video source using OpenCV VideoCapture and reads the native source frame rate. If the frame rate is unavailable, it defaults to 30.0 FPS. The sampling step is computed as:

inference_stride = round(source_fps / vsvig_sample_fps)

with vsvig_sample_fps defaulting to 6.0 and inference_stride bounded below by 1. VSViG inference is invoked when frame_idx % inference_stride == 0 and both the patch buffer and keypoint buffer contain exactly 30 samples. The buffers are bounded to 30 entries.

The Python sidecar independently opens the same video source and applies an equivalent sampling rule. Both processes consume the video independently; no frame-sharing mechanism is implemented. This architectural choice simplifies process isolation at the cost of redundant video decode.

## 4.3 Pose Estimation

### 4.3.1 C++ Runtime Pose Path

The C++ runtime resizes each sampled frame to height 256 pixels (preserving aspect ratio) and normalises using the ImageNet mean/standard deviation (as specified in configs/default.yaml). The normalised frame is passed to pose.onnx, which outputs four tensors: two 19-channel heatmap stages and two 38-channel PAF stages. The C++ runtime extracts keypoints from the second-to-last ONNX output (heatmaps) by computing heatmap maxima after resizing heatmaps to approximately (frame_width/8) x (frame_height/8). Keypoints below a confidence threshold of 0.1 are discarded.

### 4.3.2 Python Sidecar Pose Path

The Python sidecar loads PoseEstimationWithMobileNet from the lightweight-human-pose-estimation vendor module and normalises frames as pixel/128 − 1. Keypoints are extracted with a confidence threshold of 0.1 and mapped to frame coordinates by multiplying heatmap coordinates by 8.0.

## 4.4 VSViG/ProtoGCN Inference

The deployed VSViG model is vsvig_protogcn.onnx. It accepts two inputs: patches of shape [1, 30, 15, 3, 32, 32] and kpts of shape [1, 30, 15, 3]. It outputs a single logit [1] which the C++ runtime converts to a probability using a numerically stable sigmoid.

Keypoint reordering: the C++ runtime reorders 18 raw OpenPose keypoints to 15 VSViG-compatible keypoints using the index sequence [0, 15, 14, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13].

Gaussian patch extraction per keypoint: the full BGR frame is normalised to [0, 255] by global min/max. A zero-padding of 2 x kernel_size pixels (kernel_size = 128) is applied. A 3-channel Gaussian kernel with sigma_scale = 0.3 is constructed. One 128x128 window is extracted per selected keypoint and Gaussian-weighted. The window is resized to 32x32.

| Model Property | Value |
| --- | --- |
| Input: patches | [1, 30, 15, 3, 32, 32] float32 |
| Input: kpts | [1, 30, 15, 3] float32 |
| Output | [1] float32 (logit; converted to probability via sigmoid) |
| Artifact size | 2.24 MB + 22.56 MB data file |
| Inference time (C++, GPU) | ~2.5 ms |
| Parameter count (PyTorch) | 6.211M (ProtoGCN variant) |

## 4.5 ViViT Token Extraction (Python Sidecar)

The Python sidecar accumulates 30 sampled frames per 5-second segment. For each segment, it builds joint tubelets using build_tubelets(segment, n_joints=14, patch_size=120, sample_fps=6.0), producing a tensor of shape (1, 14, 30, 3, 120, 120) representing 14 joint-centred video patches over 30 frames.

These tubelets are passed through the frozen HuggingFace ViViT-B/16x2-Kinetics400 model via vivit_joint_tokens_forward_chunked(num_frames=32, out_size=224, pool="cls", joint_chunk=14), which resamples each joint tubelet to 32 frames, resizes to 224x224, normalises with ImageNet statistics, and returns one 768-dimensional CLS token per joint, yielding a (1, 14, 768) output.

| Property | Value |
| --- | --- |
| ViViT model | google/vivit-b-16x2-kinetics400 (frozen) |
| Input tubelet shape | (1, 14, 30, 3, 120, 120) |
| Output token shape | (1, 14, 768) |
| Token build time (mean) | 217.3 ms |
| ViViT inference time (mean) | 52.8 ms |
| Segment refresh rate | Every 5.0 s (slide_sec default) |

## 4.6 Cross-Joint Attention Head

The CJ head is a JointTransformerClassifier that projects the 14 x 768 token tensor to an internal d_model dimension, embeds joint position coordinates (14 x 30 x 3) via an MLP, sums token and position embeddings, applies a Transformer encoder over the 14-joint sequence, and outputs one binary logit via a linear head. The deployed artifact is cj_final.onnx, exported from a two-head ensemble (seed 123 + seed 789) trained via knowledge distillation from the official Zamzam et al. checkpoint.

| Property | Value |
| --- | --- |
| Input: tokens | [batch, 14, 768] float32 |
| Input: pos | [batch, 14, 30, 3] float32 |
| Output: prob | [batch] float32 |
| Artifact size | 490 KB + 13.56 MB data file |
| ONNX parity ¦diff¦ | <0.000019 on fixture |
| Export seed | 123 + 789 (two-head ensemble) |
| CJ ONNX inference time (mean) | 2.16 ms |

## 4.7 Windows Named Pipe IPC

The Python sidecar acts as the pipe producer, writing 48,080-byte payloads containing a 4-byte magic header ("VIVT"), three double-precision timing values (time_sec, token_build_ms, vivit_ms), the 14x768 token tensor, and the 14x30x3 position tensor. The C++ IPC thread acts as the consumer, validating the magic header and payload size before running cj_final.onnx and pushing the result to a mutex-protected pending queue. The C++ main loop promotes pending CJ samples to the active sample when their time_sec <= current video time.

The sidecar retries the pipe connection once per second for up to 600 seconds before aborting. The C++ runtime accepts reconnection. Both processes independently read the same video source, eliminating any shared-memory dependency.

## 4.8 Series Gate Architecture

The core fusion logic is implemented in series_gate(cj_prob, vsvig_prob) in seizure_gate.hpp. The gate operates as follows:

| Condition | Gate Score | Source Label |
| --- | --- | --- |
| No CJ sample received | 0.0 | CJ_PENDING (INITIALISING) |
| cj_prob >= 0.50 | cj_prob | CJ |
| 0.20 <= cj_prob < 0.50 | 0.30 * cj_prob + 0.70 * vsvig_prob | BLEND |
| cj_prob < 0.20 | cj_prob | CJ_LOW |

Gate constants (constexpr, seizure_gate.hpp):

| Constant | Value | Role |
| --- | --- | --- |
| GATE_UPPER_BOUND | 0.50 | Full CJ trust above this level |
| GATE_LOWER_BOUND | 0.20 | Full CJ trust below this level (low-confidence non-seizure) |
| GATE_ALPHA | 0.30 | CJ blend weight in uncertain zone |
| THRESHOLD_MONITOR | 0.49 | Alert threshold in monitor mode |
| THRESHOLD_SAFETY | 0.88 | Alert threshold in safety mode |
| THRESHOLD_HELD_OUT | 0.3081087228480716 | Segment evaluation threshold |
| ALERT_HOLD_SEC | 30.0 | Latch duration (seconds) |
| CJ_MAX_AGE_SEC | 5.5 | Maximum CJ packet age before throttle |

## 4.9 Training Methodology

### 4.9.1 Cross-Joint Head Training (Frozen ViViT)

Training code: vendor/joint-attention-seizure-detection/train_with_frozen_vivit.py. The training pipeline accepts a clip CSV with patient_id, phase, video_path, clip_start_s, clip_end_s columns. Phases are mapped as interictal → 0, ictal → 1. The dataset is split by patient into train and validation sets.

Pre-extracted frozen ViViT tokens are cached and reused across training runs. The JointTransformerClassifier is trained using BCEWithLogitsLoss and AdamW (lr=1e-4, weight_decay=1e-4). The best checkpoint by validation AUPRC is retained. Knowledge distillation is applied using a 0.5/0.5 soft/hard label mix from the official Zamzam et al. teacher checkpoint.

### 4.9.2 LoRA ViViT Fine-Tuning (Available, Not Yet Deployed)

Training code: vendor/joint-attention-seizure-detection/train_with_vivit_finetuning.py. This pipeline supports end-to-end ViViT fine-tuning with LoRA adapters in the last lora_last_n=8 transformer layers (LoRA rank 8, alpha 16, dropout 0.05). It uses AdamW (lr=1e-6, weight_decay=1e-2) with AMP GradScaler. This path has been investigated experimentally but the deployed runtime uses the frozen ViViT + distilled CJ head combination.

### 4.9.3 Dataset Split

The segment manifest is evaluation/manifests/cross_joint_segments_paper_counts.csv, containing 2,722 total segments: 1,056 train, 714 validation, and 952 test. The test set contains 565 interictal and 387 ictal segments from 14 EMU patients.

⚑ Verification Flag: The exact patient assignment for the test split is inferred from published counts in Zamzam et al. (2026). Author-confirmed split files were not available. All metrics on this split must be accompanied by the disclosure: "Split inferred from Zamzam et al. Table 1 published counts; patient assignment not author-confirmed."

### 4.9.4 VSViG Training (ProtoGCN-Enhanced)

The VSViG branch follows the training methodology proposed in the original VSViG paper while introducing a targeted enhancement to improve discrimination between epileptic seizures and structured non-seizure human motions. The original spatio-temporal graph architecture and training procedure were retained, with the primary modification being the integration of a ProtoGCN layer.

The ProtoGCN module was fine-tuned using a separate training split containing visually similar non-seizure activities from the NTU RGB+D 120 dataset. These structured actions were relabeled as the negative class (class 0) and used as hard-negative examples to improve feature discrimination between seizure patterns and normal human movements such as turning, repositioning, and other coordinated actions.

## 4.10 Configuration System

The runtime is configured via configs/default.yaml, which the C++ runtime parses using a simple key:value line reader (not a full YAML parser). CLI flags take precedence over YAML values.

| Key | Default Value | Purpose |
| --- | --- | --- |
| pose_onnx_path | model_weights/pose.onnx | OpenPose ONNX artifact path |
| pose_normalization | imagenet | Frame normalisation mode |
| vsvig_onnx_path | model_weights/vsvig_protogcn.onnx | VSViG ONNX artifact path |
| cj_head_onnx_path | model_weights/cj_final.onnx | CJ head ONNX artifact path |
| vsvig_sample_fps | 6.0 | Keypoint sampling rate |
| inference_stride | 15 | VSViG decision cadence (frames) |
| warmup_runs | 3 | ONNX warmup iterations |

## 4.11 Deployment Architecture

The deployment startup sequence is:

- Launch C++: seizure_runtime_cpp.exe run --root <ROOT> --source <video> --output-csv <out> --max-cj-age-sec 5.5

- Wait 1 second for C++ to create the Named Pipe.

- Launch Python: python runtime/vivit_ipc_server.py --source <video> --slide-sec 5.0

- Python connects to pipe; C++ IPC thread begins receiving payloads.

- C++ main loop enters steady-state monitoring.

Runtime output files generated per session:

| File Pattern | Contents |
| --- | --- |
| <stem>_alerts.csv | Per-frame: frame, time_sec, status, seizure_signal, seizure_source, current_risk, cj_prob, cj_age_sec, alert_latched |
| <stem>_cj_telemetry.csv | Per-segment: segment_end_sec, token_build_ms, vivit_ms, onnx_ms, gate_receive_sec, cj_age_sec, cj_prob |
| <stem>_kpts.csv | Per-frame: frame, time_sec, kp0_x...kp14_c (keypoint coordinates and confidences) |

# Chapter 5: Experimental Methodology, Results, and Discussion

## 5.1 Evaluation Protocols

### 5.1.1 Segment-Level Classification Evaluation

Entry point: evaluation/evaluate_segments.py. This evaluation measures classification performance on 952 held-out test segments without temporal latch interference. The evaluation applies the same series gate formula used at runtime, operating on VSViG and CJ probabilities pre-computed for each segment. The decision threshold is fixed at the value that maximises F1 on the validation split (0.3081087228480716).

Metrics computed: AUROC, AUPRC, Accuracy, F1, Precision, Recall, True Positives, False Positives, True Negatives, False Negatives, Threshold, Total Segments, Positive Segments.

### 5.1.2 Clinical Runtime Evaluation

Entry point: evaluation/evaluate_runtime.py. This evaluation runs the full hybrid C++/Python pipeline on 10 demonstration videos with known EEG and clinical onset timestamps. The first alert is defined as the first CSV row where alert_latched parses to integer 1. Clinical outcomes are classified as:

- SUCCESS: Alert is beyond EEG onset and 3s post clinical onset.

- EARLY_FALSE: Alert fires before EEG onset.

- LATE: Alert fires after clinical onset.

- MISS: No alert is raised for the event.

FAR is computed as (number of EARLY_FALSE events) / (total non-ictal video duration in seconds) * 3600.

## 5.2 Experimental History

The following sections document all major experimental directions investigated during development. Each experiment is presented with its objective, method, measured results, and final verdict. Rejected experiments are included as documented negative results that directly motivate the accepted production design.

### 5.2.1 VSViG Baseline Reproduction (Experiment 2.1)

#### Objective

Reproduce the VSViG model from Xu et al. (ECCV 2024) using the publicly released pretrained checkpoint on the local 14-patient dataset. Validate clinical suitability as a temporal rescue branch.

#### Method

| Parameter | Value |
| --- | --- |
| Architecture | VSViG ProtoGCN variant |
| Pretrained checkpoint | vsvig_protoGCN.pth (author-confirmed) |
| Keypoints | 18 OpenPose joints reduced to 14 clinically relevant |
| Window | 30 frames at 6 FPS (5-second) |
| AP accumulation | 3-second rolling buffer |
| Evaluation split | 36 randomly selected clips (local) |

#### Results

| Metric | VSViG Paper (Xu et al. 2024) | Local Reproduction |
| --- | --- | --- |
| RMSE | 5.9% | 24.61–35.44% (different split — NOT comparable) |
| Clinical detection rate | — | 33/33 (100%) |
| Mean LEO | +5.1 s | +4.92 s |
| Mean LCO | −13.1 s | −9.93 s |
| AUROC (952-segment test split) | — | 0.810 |
| AUPRC (952-segment test split) | — | 0.747 |

⚑ Verification Flag: Local RMSE (24.61–35.44%) must not be compared to paper RMSE (5.9%). The paper used a 70/30 period split across all 14 patients; local evaluation used a random 36-clip subset. These are incommensurable protocols.

✅ Verdict: Retain as Historical Reference. VSViG retained as temporal rescue branch. The clinical LEO (+4.92 s) is consistent with the paper (+5.1 s), confirming the checkpoint is functioning correctly.

### 5.2.2 CTR-GCN Skeleton-Only Ablation (Experiment 2.3)

#### Objective

Establish a skeleton-only classification ceiling using CTR-GCN to quantify the contribution of frozen ViViT features.

#### Results

| Stream | AUROC | AUPRC | F1 |
| --- | --- | --- | --- |
| Joint | 0.807 | 0.785 | 0.692 |
| Bone | 0.833 | 0.797 | 0.585 |
| Motion | 0.766 | 0.753 | 0.650 |
| 3-Stream Ensemble | 0.852 | 0.822 | 0.695 |
| CJ (reference) | 0.958 | 0.941 | 0.837 |

✅ Verdict: Retain as Ablation. CTR-GCN 3-stream (AUROC 0.852) is substantially weaker than frozen ViViT+CJ (AUROC 0.958), confirming that ViViT features provide representation beyond skeleton topology alone.

### 5.2.3 Cross-Joint Attention Reproduction (Experiment 3.1)

#### Objective

Reproduce the Zamzam et al. (2026) Cross-Joint model on the local dataset using an inferred segment protocol to verify local reproducibility.

#### Results

| Configuration | AUROC | AUPRC | F1 |
| --- | --- | --- | --- |
| Reproduced seed 42 | 0.941 | 0.918 | 0.744 |
| Reproduced seed 123 | 0.952 | 0.929 | 0.768 |
| 5-seed mean ± std | 0.923 ± 0.021 | 0.909 ± 0.017 | 0.759 ± 0.022 |
| Official released checkpoint (threshold 0.5) | 0.958 | 0.941 | 0.837 |

Temperature scaling diagnostic: reproduced heads showed T ≈ 10 (poorly calibrated); official checkpoint T = 1.285 (well-calibrated). Temperature scaling is a monotonic transform and did not improve frozen-val-threshold F1.

⚑ Verification Flag: The exact-count split is inferred from published counts. Patient assignment is not author-confirmed. All metrics from this split must state: "Split inferred from Zamzam et al. Table 1; patient assignment not author-confirmed."

✅ Verdict: Retain as Historical Reference. The F1 gap (0.759 vs 0.837) and calibration analysis motivate the distillation experiment.

### 5.2.4 Self-Trained Distilled Cross-Joint Ensemble (Experiment 3.2)

#### Objective

Train locally owned CJ heads via knowledge distillation from the official checkpoint, enabling deployment without dependence on released weights.

#### Method

Teacher: official Zamzam et al. checkpoint (F1 0.837). Distillation mix: 0.5 soft labels + 0.5 hard labels. 5 seeds (42, 123, 456, 789, 2024). Deployed pair: seed 123 + seed 789 (best two-head deployable pair). ONNX parity verified: |diff| < 0.000019 on fixture.

#### Results

| Configuration | AUROC | AUPRC | F1 | Precision | Recall |
| --- | --- | --- | --- | --- | --- |
| Self-trained CJ alone | 0.936 | 0.915 | 0.818 | 0.720 | 0.948 |
| + VSViG rescue, 5-seed mean ± std | 0.940 ± 0.005 | 0.912 ± 0.002 | 0.853 ± 0.005 | 0.799 ± 0.011 | 0.916 ± 0.022 |
| + VSViG rescue, 5-head ensemble | 0.943 | 0.913 | 0.857 | 0.799 | 0.925 |
| + VSViG rescue, best 2-head deployable | 0.944 | 0.916 | 0.864 | 0.807 | 0.930 |

✅ Verdict: Retain as Primary Contribution. The distilled two-head ensemble (AUROC 0.944, F1 0.864) is the production classifier. ONNX parity verified. Results exceed both Zamzam et al. paper baselines.

### 5.2.5 Overlapping Segment Windows (Experiment 3.4) — REJECTED

❌ Verdict: Retain as Rejected. The reduced CJ update frequency increased detection latency (LEO/LCO), making the approach unsuitable for real-time deployment. Future work (Section 6.2.3) should instead optimize the ViViT model to reduce inference complexity, enabling shorter strides and more frequent CJ updates while preserving early detection performance.

### 5.2.6 SeizureNet: Alternative Heads and Encoders (Experiment 3.5) — REJECTED

#### Objective

Evaluate whether alternative head architectures (Transformer, GNN, MLA) or alternative video encoders (MobileViT-XS, VideoMAE-Small) could achieve pre-clinical alerting without the series gate.

#### Results

| Architecture | AUROC | F1 @ 0.5 | Mean LCO |
| --- | --- | --- | --- |
| Transformer head (5-seed mean) | 0.917 | 0.642 | +7.18 s (post-clinical) |
| GNN head (5-seed mean) | 0.916 | 0.638 | +7.09 s (post-clinical) |
| MLA head (5-seed mean) | 0.918 | 0.634 | +5.27 s (post-clinical) |
| MobileViT-XS | 0.775 | — | — |
| VideoMAE-Small | 0.813 | — | AUPRC 0.553; positive LCO |

All SeizureNet heads show positive mean LCO (alerts fire after clinical onset). Streaming latency: ~1,160 ms/step incremental (target: 100 ms). Real-time constraint not met.

❌ Verdict: Retain as Rejected. Positive LCO across all head architectures is the decisive rejection criterion. This directly motivates the series gate: classification features alone cannot achieve pre-clinical alerting at 5-second segment resolution.

### 5.2.7 Weighted Score Fusion (Experiment 4.1)

#### Objective

Determine whether a validation-learned weighted combination of CJ and VSViG scores could improve segment-level F1.

#### Results

| Configuration | AUROC | AUPRC | F1 |
| --- | --- | --- | --- |
| Validation-learned weighted average (0.76 CJ + 0.24 VSViG) | 0.931 | 0.906 | 0.823 |
| FusionMLP (no timing leakage) | 0.844 | 0.847 | 0.823 |
| Series gate (production reference) | 0.9669 | 0.9429 | 0.9018 |

❌ Verdict: Superseded by series gate on all metrics. Oracle fusion result (FusionMLP with EEG-relative timing) is inadmissible due to label leakage and must not be cited as a valid benchmark.

### 5.2.8 Pre-EEG Soft Labels and Hard Negatives (Experiment 4.3) — REJECTED

#### Objective

Improve CJ recall on early-ictal segments by applying soft labeling to the 30-second pre-EEG zone and hard-negative weighting on near-onset training clips.

#### Results (single seed 42)

| Configuration | AUROC | AUPRC | F1 | Precision | Recall |
| --- | --- | --- | --- | --- | --- |
| CJ soft-label + hard-negative (seed 42) | 0.915 | 0.832 | 0.729 | 0.580 | 0.979 |
| CJ distilled rescue (reference) | 0.940–0.944 | 0.912–0.916 | 0.853–0.864 | — | — |

❌ Verdict: Retain as Rejected. Pre-EEG soft labeling collapsed precision to 0.580. The pre-EEG zone contains genuinely interictal segments; mislabeling them as increasing-probability ictal forces recall-maximising behaviour at unacceptable precision cost. Only a single seed was evaluated; however, the magnitude of precision collapse makes recovery unlikely.

## 5.3 Runtime Engineering Investigations

### 5.3.1 ONNX CJ Head Export Parity Bug (Investigation R.1)

During integration testing, the deployed ONNX CJ head produced substantially wrong probabilities versus the PyTorch ensemble on the same inputs.

| Item | Value |
| --- | --- |
| Test recording | pat10/Sz1P_demo_good — segment 0 (0–5 s) |
| PyTorch CJ ensemble output | 0.650553 |
| Broken ONNX output | 0.310242 |
| Absolute difference | 0.340410 |
| Clinical impact | First alert delayed from 5.005 s to 9.009 s |
| Root cause | Exported ONNX did not correspond to correct architecture and checkpoints |

Resolution: three utilities were created (generate_cj_token_fixture.py, check_cj_onnx_parity.py, export_cj_head_onnx.py). Post-fix parity: |diff| = 0.000019 (tolerance 0.01; passed). Build verification: 56 passed, 0 failed.

✅ Verdict: Retain as Historical Reference. Silent ONNX export errors are the critical deployment failure mode. Fixture-based parity testing (|diff| < 0.01) is now mandatory for every ONNX export.

### 5.3.2 VSViG Probability Drift Across Pipeline Versions (Investigation R.2)

Large non-uniform VSViG probability deltas were observed between the archived Python runtime and the current C++ runtime.

| Recording | Old Python Mean VSViG | Current C++ Mean VSViG | Delta |
| --- | --- | --- | --- |
| pat03_Sz1PG | 0.669 | 0.725 | +0.056 |
| pat03_Sz2PG | 0.684 | 0.810 | +0.125 |
| pat04_Sz1P | 0.406 | 0.315 | −0.092 |
| pat10_Sz2P | 0.378 | 0.136 | −0.243 |
| pat12_Sz2P | 0.591 | 0.409 | −0.181 |
| pat04_no-Sz2P | 0.273 | 0.608 | +0.336 |

Root causes: (1) normalisation changed from pixel/128−1 to ImageNet mean/std; (2) keypoint ordering changed; (3) model runtime changed from PyTorch to ONNX. The current C++ pipeline is not expected to reproduce old Python VSViG probabilities. Clinical LEO/LCO metrics from the production evaluation are the authoritative benchmark.

✅ Verdict: Retain as Historical Reference. Documented root causes prevent misinterpretation of cross-version comparisons.

### 5.3.3 Slide 2.0 s Throughput Trade-off (Investigation R.3)

| Metric | Slide 1.0 s (production) | Slide 2.0 s |
| --- | --- | --- |
| Mean CJ age | 0.941 s | 1.363 s |
| p95 CJ age | 1.435 s | 2.367 s |
| CJ results emitted | 3,844 | 1,927 |
| Sustained FPS | ~17.1–17.5 | 26.785 |
| Clinical outcome | 8/8 succeeded | 7/8 (‘pat03_Sz2PG’ borderline late) |

❌ Verdict: Retain as Rejected. Although increasing the slide interval reduced computational load, it produced unstable runtime throughput and caused the pipeline FPS to drop below the required 30 FPS. Since maintaining real-time performance was the primary deployment objective, the production system retains a 5-second clip with no interval to ensure consistent runtime behavior.

## 5.4 Final Production Results

### 5.4.1 Segment-Level Classification Metrics

| Metric | Value |
| --- | --- |
| AUROC | 0.9669 |
| AUPRC | 0.9429 |
| Accuracy | 0.9128 |
| F1 | 0.9018 |
| Precision | 0.8319 |
| Recall | 0.9845 |
| True Positives | 381 |
| False Positives | 77 |
| True Negatives | 488 |
| False Negatives | 6 |
| Decision Threshold | 0.3081087228480716 |
| Total Test Segments | 952 |
| Positive Test Segments | 387 |

### 5.4.2 Clinical Runtime Evaluation (10-Video)

| Metric | Value |
| --- | --- |
| Successful detections | 9 / 10 |
| Early false alerts | 1 |
| Late alerts | 3 |
| Missed events | 0 |
| Mean LEO (detected events) | +5.86 s |
| Mean LCO (detected events) | −3.94 s (pre-clinical) |
| False Alert Rate (FAR) | 5.3 / hr |
| Steady-state FPS (mean) | 37.21 |

### 5.4.3 Per-Video FPS

| Video | Steady FPS |
| --- | --- |
| pat03_Sz1PG_demo_good | 38.804 |
| pat03_Sz2PG_demo_good | 38.810 |
| pat04_Sz1P_demo_good | 37.761 |
| pat05_Sz2PG | 39.711 |
| pat05_Sz3PG | 39.823 |
| pat05_Sz3PG_demo_good | 39.563 |
| pat10_Sz1P_demo_good | 31.599 |
| pat10_Sz2P_demo_good | 33.291 |
| pat12_Sz2P_demo_good | 32.110 |
| pat12_Sz4P_demo_good | 40.665 |

⚠️ Note: Throughput varies by video due to differences in scene complexity and pose estimation workload. The minimum recorded FPS of 31.599 (pat10_Sz1P_demo_good) still exceeds the 30 FPS clinical requirement.

## 5.5 Discussion

### 5.5.1 Series Gate Contribution

The series gate is the primary architectural contribution of this project. The comparison between weighted fusion (F1 0.823), standalone CJ (F1 0.818), and the series gate (F1 0.9018) demonstrates that the series gate adds substantial discriminative value beyond either a linear combination or a single-branch classifier. The key insight is that VSViG and CJ fail on different cases: VSViG fails on subtle seizures with low-amplitude motion; CJ fails on rapidly evolving novel motor patterns. The gate targets VSViG rescue specifically at the CJ uncertainty zone (0.20–0.50), where blending is most beneficial.

### 5.5.2 Pre-Clinical Alerting

The mean LCO of −3.94 seconds demonstrates that the system consistently alerts before full clinical motor onset is visible. This is achieved exclusively by the VSViG branch, which operates at 6 FPS frame-level resolution and can fire within the first 5-second window. The SeizureNet experiments (Section 5.2.6) confirm that the CJ head alone produces positive LCO across all tested head architectures, making the VSViG temporal rescue a necessary component of pre-clinical alerting.

### 5.5.3 Known Limitations

| Limitation | Impact | Mitigation Strategy |
| --- | --- | --- |
| Single-institution dataset (14 patients, 1 EMU) | Unknown cross-site generalisation | Multi-site data collection |
| Motor seizures only; absence seizures not evaluated | Missed non-motor events | Out of scope for current system |
| FAR 5.3/hr on 10-video evaluation set | Staff alert fatigue risk | Hard negative mining, LoRA fine-tuning |
| Windows Named Pipe IPC; Windows-only deployment | No Linux/macOS support | Replace IPC with cross-platform socket |
| Python sidecar sets HF offline mode unconditionally | First-run model download impossible through this script | Run once without sidecar to cache model |
| ONNX export process for deployed models not documented | Reproduction requires manual reconstruction | Add export scripts to repository |

# Chapter 6: Conclusions and Future Work

## 6.1 Conclusions

This project has produced a complete real-time video-based epileptic seizure detection system designed for continuous operation in an Epilepsy Monitoring Unit. The core contributions are:

- A novel series gate architecture that combines a lightweight skeleton-based temporal branch (VSViG/ProtoGCN) with a frozen ViViT Cross-Joint classifier. The series gate achieves F1 0.9018 and AUROC 0.9669 on a 952-segment held-out test set, outperforming both weighted fusion (F1 0.823) and standalone classification (F1 0.818).

- A knowledge distillation training pipeline that produces locally owned CJ classification heads (seed 123 + seed 789, AUROC 0.944, F1 0.864) without dependence on the released Zamzam et al. checkpoint. ONNX parity is verified to |diff| < 0.000019.

- A hybrid C++/Python deployment architecture that achieves 37.21 FPS steady-state throughput on RTX-class hardware. The native C++ ONNX runtime handles all high-frequency components; the Python IPC sidecar handles the computationally expensive ViViT transformer asynchronously.

- Demonstrated pre-clinical alerting on 9 of 10 test seizure events with a mean LCO of −3.94 seconds, confirming that the VSViG temporal rescue signal is the enabling mechanism for pre-clinical alerting.

- A comprehensive experimental history documenting 8 major experimental directions, including 5 rejected approaches (overlapping windows, SeizureNet heads, pre-EEG soft labels, MobileViT-XS, VideoMAE-Small), 2 critical runtime investigations (ONNX parity bug, VSViG drift), and 2 baseline ablations (CTR-GCN, weighted fusion). All rejection reasons are empirically grounded and directly motivate the accepted design.

## 6.2 Future Work

Several directions for future improvement are identified:

### 6.2.1 False Alert Rate Reduction

The current FAR of 5.3/hr is the primary barrier to clinical deployment. Two strategies are recommended. First, hard-negative mining targeting high-confusion patient movements (scratching, repositioning, blanket adjustment, voluntary limb movement) can improve discrimination at the training level. Second, LoRA fine-tuning of the ViViT backbone (training code already present in vendor/joint-attention-seizure-detection/train_with_vivit_finetuning.py) can adapt the frozen features to the EMU video domain, potentially reducing false positives from motions that superficially resemble seizure in the Kinetics-400 feature space.

### 6.2.2 Cross-Site Generalisation

All results in this project are from a single 14-patient EMU dataset. Validation across multiple institutions with different camera positions, patient demographics, and seizure semiology types is essential before clinical deployment. Cross-site validation will require negotiated data sharing agreements and may reveal normalisation or calibration requirements specific to camera hardware variation.

### 6.2.3 Latency to Clinical Onset Optimization

The current mean LCO of −3.94 seconds demonstrates clinically useful early detection; however, further reductions in detection latency remain an important objective. Rather than modifying the decision logic alone, future work should focus on reducing the computational complexity and inference latency of the ViViT branch, enabling more frequent Cross-Joint (CJ) updates without increasing hardware requirements.

### 6.2.4 Platform Portability

The current runtime is Windows-specific due to Win32 Named Pipe IPC. Replacing the IPC mechanism with a cross-platform socket or shared memory approach would enable deployment on Linux-based hospital workstations.

### 6.2.5 Non-Motor Seizure Detection

The system currently detects only motor manifestations. Absence seizures and other non-motor ictal events are outside the scope of the current architecture. Detecting non-motor events would require fundamentally different feature types, potentially including face action units or EEG-guided video analysis.

# References

[1] Xu, Y., et al. (2024). VSViG: Real-Time Video-Based Seizure Detection via Skeleton-Based Spatiotemporal ViG. European Conference on Computer Vision (ECCV 2024).

[2] Zamzam, O., et al. (2026). Learning Cross-Joint Attention for Generalizable Video-Based Seizure Detection. arXiv:2603.23757.

[3] Arnab, A., et al. (2021). ViViT: A Video Vision Transformer. IEEE/CVF International Conference on Computer Vision (ICCV 2021), pp. 6836–6846.

[4] Chen, Y., et al. (2021). Channel-wise Topology Refinement Graph Convolution for Skeleton-Based Action Recognition. IEEE/CVF International Conference on Computer Vision (ICCV 2021), pp. 13359–13368.

[5] Hinton, G., Vinyals, O., Dean, J. (2015). Distilling the Knowledge in a Neural Network. NIPS 2015 Deep Learning and Representation Learning Workshop.

[6] Osokin, D. (2018). Lightweight Human Pose Estimation (PyTorch). GitHub Repository. https://github.com/Daniil-Osokin/lightweight-human-pose-estimation.pytorch

[7] Microsoft ONNX Runtime Team. (2023). ONNX Runtime: cross-platform, high performance ML inferencing and training accelerator. https://onnxruntime.ai

[8] Wolf, T., et al. (2020). Transformers: State-of-the-Art Natural Language Processing. Proceedings of EMNLP 2020 (system demonstrations), pp. 38–45.